Wire Rope Non-Destructive Testing

(Computerize Magnetic Wire Rope Testing And Diagnosing System)

MTC Shanghai  Qiehua Virtual Instrument Co., Ltd

Modification History

| Date | Version | Description |
| --- | --- | --- |
| 31 August 2001 | A0 | Initial Release |

###### Current Modification Details

| Revision | Revision made |
| --- | --- |
| 0 |  |

Prepared by	:									Date:

Hu Qiang, Project Engineer

Reviewed by:									Date:

He DongHuan, Com. Director

Authorized by:									Date:

Dong XinHua, General Director

Hard copies of this document bearing no controlled stamp are for reference only. The electronic version of this document bears no controlled stamp.

|  | #### CONTENTS |  |
| --- | --- | --- |
| Section | Description | ##### Page |
| 1. | Abstract | 5 |
| 2. | Introduction | 6 |
| 3. | Purpose of Nondestructive Testing | 7 |
| 4. | Performance Criteria | 8 |
| 5. | Principles of Operation | 10 |
| 6 | Specifications | 13 |
| 7. | Certification, National Award & Patent | 15 |
| 8. | Main World Producers of Non-Destructive Magnetic Wire Rope Test Apparatus | 16 |
| Appendix A | Reference | 17 |

1.	Abstract

The art of wire rope inspection has progressed rapidly during the past few years. Electromagnetic instruments have been developed over the past 90 years. The advancement of Hall Effect sensor and computerization can reliably test wire ropes in service and can remedy the shortcomings of visual wire rope inspection methods.

A statistical analysis made in US of over 8000 laboratory and field test records revealed some interesting facts on the condition of wire ropes in services.

- Approximately 10 percent of all ropes considered showed a strength loss of over 15 percent; more than 2 percent of the ropes had lost over 30 percent of their nominal strength. In other words, while still in service, 10 percent of all ropes were in an unacceptable and potentially hazardous condition, and 2 percent of the ropes were in an extremely dangerous condition.

- Conversely, more than 70 percent of all ropes in the sample were removed from service with little or no strength loss. This finding suggests that only a very small percentage of all ropes was replaced in a timely manner.

- Introduction

MTC is an Award Winning Non Destructive Testing (NDT) system based on patent pending electromagnetic sensing technology and mathematical model invented by Shanghai Yinhua Wire Ropes Testing Co. Ltd. The combination results a reliable and precise identification and quantitative analysis of the degradation along a rope. The principal deterioration modes of wire rope can be categorized as follows.

  - Loss of Metallic Cross-Sectional Area (LMA)

Rubbing along floors or other surfaces causes external abrasion. Internal abrasion is caused by nicking, high pressures, or poor lubrication. Corrosion (external and internal) is caused by environmental conditions or poor lubrication.

  - Localized Faults (LF)

Broken wires are caused by fatigue, plastic wear, martensitic embrittlement, or mechanical damage. Kinks and other mechanical damage may also occur. Although many nondestructive test procedures, employing radiation and optical, acoustical, and mechanical methods, have been proposed and tried in the past, at the present time, only visual and electromagnetic test methods are practical.

MTC NDT Wire Rope Testing and Diagnosing System allow simultaneous LMA and LF inspection. The equipment was launched in 1997, and received numerous recommendations from customers, including Shanghai Baosteel Group, Yangtse River Three Gorges Engineering, Shanghai Waigaoqiao Free Trade Zone Stevedoring Co., Shanghai Shipyard, Shandong Shanjiachum Coal Mine, Suzhou Amusement Land, Shanxi Luan Coal Mine, Zhongyuan Oil Field, Shidongkou Power Engineering Co., Tongling Yangtse River Stay-cable Bridge and so forth.

Shanghai Yinhua Wire Rope Testing Co. Ltd has a strong R&D team together with the support from Scientific & Techonolgy Research Centre PRC, Shanghai JiaoTong University, Tonggi University and OSC GmbH Germany.

## Purposes of Nondestructive Testing

Nondestructive tests in great variety are in worldwide use to:

  - Ensure the integrity and reliability of a product.

  - Avoid failures, prevent accidents and save human life.

  - Make a profit for the user.

  - Ensure customer satisfaction and maintain the manufacturer’s reputation.

  - Aid in better product design.

  - Control manufacturing processes.

  - Lower manufacturing costs.

  - Maintain uniform quality level.

  - Ensure operational readiness.

4.	Performance Criteria

4.1	Resolution

The resolution of a transducer is measured as the smallest distance between flaws for which the transducer provides distinctly separate flaw indications. Resolving power is defined as the reciprocal of resolution.

4.2	Quantitative Resolution

The quantitative resolution is the required minimum length of a uniform flaw for which the sensor provides an accurate quantitative measurement of a rope’s change of metallic area (CMA) within a predefined small error limit. Quantitative resolving power is defined as the reciprocal of the quantitative resolution.

4.3	Penetration

The penetration of a transducer is measured by the ratio of the signal amplitude, caused by an internal flaw, to signal amplitude, caused by an identical surface flaw. This ration is also called the Penetration Ratio.

4.4	Signal to Noise Ratio

Only test signal components, which are caused by rope defects, are of interest. That part of the test signal that is not caused by defects is considered as noise. The signal to noise ratio is defined as the amplitude ratio of the defect related signal component to noise.

4.5	Flaw Detectability

Flaw detectability is defined as the smallest cross-sectional area change which the sensor can detect. Note that flaw detectability is strictly a function of and intimately related to the signal to noise ratio. A signal to noise ratio greater than one is required for flaw detection.

4.6	Sensitivity

The sensitivity is defined as the signal amplitude caused by a pre-determined flaw. In designing rope test instruments, sensitivity specifications are usually meaningless, as it can easily be increased by increasing the gain of the signal amplifier.

4.7	Repeatability

Many sensors used for rope inspection are either subdivided or otherwise not rotationally symmetric. Hence, noise as well as flaw signals depends on the angular position of the rope with respect to the sensor head, and complete repeatability of signals cannot be assured for some instruments.

4.8	Magnetic Interference

Since insulating materials for magnetic fields do not exist, magnetic flux is difficult to contain. All electromagnetic rope test instruments are surrounded by a magnetic leakage field. Therefore, foreign ferrous objects, such as steel beams, pipes, steel floors, or tightly spaced ropes, in the immediately vicinity of the test instrument can influence the test results. Preventing lateral movement of foreign steel objects – for instance, of adjacent ropes – relative to the sense head eliminates or minimizes problems caused by interference.

4.9	Weight and Size

For optimum performance, the magnetizer has to drive the rope into magnetic saturation under all operating conditions. To reduce the weight of the sensor head without sacrificing performance, advanced instruments use ultra powerful rare-earth permanent magnets.

4.10	Operating Convenience

For on-site field inspections, the operating convenience of an instrument is very important. Since electric power is not always easily accessible, advanced instruments are battery operated and with distance counter, data acquisition system, user-friendly Man Machine Interface (MMI).

- Principles of Operation

LF/LMA Quantitative Analysis Software

A/D Converter

Computer

Monitor

Signal

Generation					Coder

Printer

Process

Steel Wire Rope

Sensor Head

Assemble

Figure 1

Functional Block of a MTC NDT Wire Rope Testing & Diagnosing System

5.1	The strong permanent magnets installed in the Sensor Head Assemble supply a constant flux that magnetizes a length of steel wire rope (0.8T) and build up magnetizing circuit as it passes through the sensor head. The saturating magnetic field enables the magnetic sensor to visualize the mechanical anomalies present in the wire rope. The process is somewhat similar to making an NDT examination of a human body with X-rays, where density variations of the patient are made visible by greater or lesser absorption of these rays.

5.2	The magnetic flux leakage created by a discontinuity in the rope, such as broken wire and surface, corrosion, will be focused by the magnetic focus rings and detected by the array Hall effect devices. The local flaw (LF) can then be quantitatively measured. The total axial magnetic flux created by the amount of material missing from location along the wire rope, such as corrosion, deformation, abrasion spots, will be received due to putting the Hall effect sensors on the balance point of magnetic bridge. The loss of metallic cross-sectional area (LMA) can then be also quantitatively measured.

5.4	The coder measured the length of the wire rope under test, and generates continuous pulse to the computer for the recording of distance traveled.

5.5	The Signal Generation Process further enlarge, filter and rectify the detected analogue signal to the A/D converter where it is digitized (12 Bits) and sent to the conjoint computer for data processing and storage.

Disturb

Testing Process

Modeling

Specification of broken wire				Model					Signature Feature of Broken

Wire Testing

Broken Wire Testing

State & Specification

Estimation

Change & Error Engendered

Error Accumulation

Broken Wire Detection

Result

Estimation of Broken Wire

###### Figure 2

###### MTC LF Quantitative Testing Software Basic Module

5.6	With intensive Probability of Detection (POD) experiments, MTC envision a set of curves. These curves demonstrate that a certain size discontinuity can be detected with a certain probability at a given confidence level. Base on this statistical concept, test matrices are developed that exercise all potential variable of each test and derive intelligent decision with high level of confidence (≧92%).

5.7	A damaged wire rope would generate mutation signal, containing the following information:

- leakage flux around a local fault (LF);

- change in the rope’s magnetic impedance, due to a loss in the magnetic cross sectional area (lma); and

- change in the magnetic flux value of the magnetic circuit in the sensor head.

5.8	The complex signal generated would be in form of variation of signal peak, wave width and shapes. These signals will be adjusted, and matched by the MTC software together with the pre-set steel wire rope testing parameters. A quantitative estimation result would be generated for verification through the Human Machine Interface.

5.9	Due to complicated construction, manufacturing error and different condition of wire rope, it is recommended that the MTC NDT Wire Rope Testing and Diagnosing System should be used after new installation of the wire rope. The corresponding test record obtained from regular on-going testing should be used to compare with the initial parameters. This practice would bring the system to arrive the most accurate quantitative result.

- Specifications

M	T	C  -  □	□

Pillow diameter related to tested wire rope diameter

MC	Φ < 8mm

S   8 ≦ Φ ≦ 19mm

M	 19 <Φ < 60mm

L    60 ≦ Φ ≦ 88mm

XL   Φ > 88mm

Computerize

Testing and Diagnosing

Magnetic Principle

6.1	The MTC NDT Wire Rope Testing and Diagnosing System consists of the following items:

- Sensor Head Assembly comprising

  - Magnet Assembly

  - Sensor Assembly

  - Distance Counter Wheel Assembly

  - A/D Converter

- Signal Processing Assembly including

  - Portable computer

  - MTC Wire Rope Testing & Diagnosing (WRTD) Software

  - Universal printer

- Basic Parameters

Magnetizing Power 				> 0.8T

A/D Converter					4 –16 channel, 12 bit, 5V ±10% d.c/20mA

Connectors						JB1399

Computer						CPU 546 or above

Internal RAM > 4M

2 serial ports, 1 parallel port

Display: SVGA (640 x 480 pixel) or above

- Performance

Rope Sizes						Φ 1.6 to 200mm

Rope Speed						0.003 to 6m/sec

Test Signals						LF and LMA signal amplitudes

Independent of rope speed

LF Detection

Qualitative Detectability			100%

Quantitative Detectability			92% of broken wires

Amount of broken wires allow error of one wire independent of wire diameter

LMA Detection

Quantitative Detectability	±0.05% of changes of metallic cross sectional area

LF/LMA Measure Position	±2cm of LF/LMA

LF/LMA Test Repeatability	tendency to 0

Best Gap between Wire Rope	2 to 6mm

And Pillow of Sensor Head

Environmental Conditions	-10 ℃ to 55 ℃ (Operating temperature)

≦95% (Relative Humidity)

7.	Certification, National Award & Patent

7.1	1998, China State Bureau of Technical Supervision authorizes, National Center of Testing Technology, Shanghai, PRC verified and awarded certificate to MTC NDT Testing and Diagnosing System.

7.2	1998, National Science and Technology Research Center, Institute of Scientific & Technical Information of Shanghai, PRC approved MTC performance and functions are international advance and outstrip the standard practice of the American Society for Testing and Materials (ASTM) E1571-96.

7.3	1998, Grade A New High Technology Achievement Certificate awarded by the Government of Shanghai, PRC.

7.4	Theoretical & Methodology Invention Patent: ZL 92115277.9

7.5	Application of MTC Model Patent:	ZL 00216937.1

8.	Main World Producers of Non-Destructive Magnetic Wire Rope Test Apparatus

| Producer and Country | Detecting Sensors Type Technique |  | Recording & Data Processing | Designation of Instrument |
| --- | --- | --- | --- | --- |
| AGH Poland | Area/ Local faults | Hall effect/ inductive coils | Fully digital, PCMCIA Memory | MD120 and GM series of heads |
| AATS South Africa | Area/ Local faults | Hall effect | Dedicated computer | AATS Model 817 |
| British Coal, Great Britain | Area/ Local faults | Coils and integrator | Gould termite paper chart recorder | Ropescan |
| DMT Germany | Area/ Local faults | Coils with integrator | Built-in chart recorder, PC card | RTI 1 |
| Dr. Brandt Germany | Area/ Local faults | Coils with integrator | Built-in chart recorder, Gould | SPM-1, SPM-20 SPR |
| Druk Pak AG/EMSA Switzerland/Holland | Local faults | Hall effect | Paper recorder WK-150 | Hand “Cable Spy” |
| E Kündig SA Switzerland | Local faults | Inductive coils | Gould-Brush chart recorder | PMK 75 and others |
| ETH Zurish Switzerland | Local faults | Coils | No data | No data |
| Halec SA France | Local faults | Coils | Gould chart recorder | Cable Test Halec AS |
| Health and Sherwood Canada | Area/ Local faults | Hall effect | Digital, computer processing | Magnograph II |
| Hitachi Building System Eng and Service, Japan | Area | AC coils | Prototype | AC Elevator Rope Tester |
| Hitachi Building System Eng and Service, Japan | Local faults | DC circuit, coils | Prototype | Magnetic Defect Sensor |
| Intron Plus Ltd Russia | Area/ Local faults | Hall effect | Build in Data Logger, computer processing | INTROS, MH & F series |
| Kanatop Electro-Mech Plant, Ukraine | Area only | AC electromagnet | External chart recorder | IISK-5 |
| Laboratory Roman Martyna, Poland | Area/ Local faults | Hall effect | Analogue, termite chart recorder | LRM-MH LRM-MR |
| Meraster, Poland | Area/ Local faults | Hall effect/ inductive coils | Fully digital, PCMCIA memory | MD120 and GP series of heads |
| NDT Technologies USA | Area/ Local faults | Coils and integrator | DSP and /or built in chart recorder | LMA-125, 175,250, LMA-test |
| RAU | Area/ Local faults | Coils & hall flux reversal | Chart | RAU |
| Rotesco, Canada | Area/ Local faults | Flux gate/ inductive coils | Chart recorder/ computer hard drive | Rotescograph 2D and 2C-TAG88M |
| Shanghai Maritime University, PRC | Area | Flux gate | Prototype |  |
| Technical University Stuttgart, Germany | Local faults | Magneto-inductive | No data |  |
| TUV UK Ltd, Germany | Local faults | Coils? | Gould chart recorder | Wire Rope test |
| VVUU, Czech Republic | Area/ Local faults | Hall effect | Fully digital | MID-series |
| Wire Rope Testers, Inc. USA | Local faults | Inductive coils | Digital, laptop PC, Win 95 | The Rope Tester |
| ZEG-Tychy, Poland | Area/ Local faults | Hall effect/ coils | Analogue, termite chart recorder | DLS (not produced) |

APPENDIX A

REFERENCES:

- Dong Xinhua, Kang Yihua, Lu Nanyin “NDT Solution, MTC Novel, Computerize, Dual Quantitative Function, Electromagnetic Wire Rope Testing and Diagnosing System”

- Herbert R. Weischedel “The Inspection of Wire Ropes in Services: A Critical Review”

- American Society for Nondestructive Testing, Inc. “ Introduction to Nondestructive Testing”

- Robert H Grills, “ Probability of Detection – An NDT Solution”

- Health & Safety Executive, UK “ Wire Rope  Non-Destructive Testing – Survey of Instrument Manufacturers”

- KML Railway Engineering Solutions Ltd.

“Wire Rope Non – Destructive Testing”